Headless_Task-Grounded_Pick-and-Place_in_Gazebo — implementation report

# Task-grounded pick-and-place, headless, from spec to a testable launch file

What was built against doc/headless_pick_and_place_specification.md, step by step, with the verification used for each step, every bug found along the way, and the 2026-09-22 addendum adding a single ros2 launch entry point for manager review.

Sandbox  ~/tomislav/robotics/mycobi_R6A_tomislav_22_09_2026/Headless_Task-Grounded_Pick-and-Place_in_Gazebo

Host  Ubuntu WSL2, ~4.9 GB usable RAM, CPU-only

Reproduced via  the gazebo_to_lerobot Docker container (Ubuntu 24.04, ROS2 Jazzy, Gazebo Harmonic)

Commit  5c92a565 on mycobi_R6A_tomislav_22_09_2026_tomislav_branch

Written  2026-09-22

Updated  2026-10-03 (Sec.16–30, GUI demo on WSL2)

# Overview

This document is the implementation report for headless_pick_and_place_specification.md — the file in this same directory that was treated as a binding specification of design and verification *goals*, not of implementation details. Per the governing instruction under which this work was done, specific programming choices in that document were free to be replaced with better solutions; every goal had to be either achieved or explicitly reported as a problem.

The short version: every numbered step in the specification (Parts 1-8) was implemented and verified, twenty LeRobot-format episodes were recorded and split into a 15-episode training set and a 5-episode held-out-camera validation set, and the pipeline is fully self-verifying (a CSV-based pass/fail check runs at the end of every episode). The one load-bearing deviation is in the grasp mechanism itself: after a deliberate, bounded physics experiment produced no friction hold, the block is carried by a Gazebo DetachableJoint weld triggered on gripper-close and released on gripper-open — labelled SIMULATED ATTACHMENT, NOT a physical grasp everywhere it appears, in code comments, log lines, and every episode's own metadata. Four bugs were fixed in the arm-and-gripper sequencing path, three more in the recording-batch infrastructure, three more in the demo-launch orchestrator added on 2026-09-22 (Sections 13-15), and four are flagged as worth reporting upstream.

Addendum, 2026-09-22 (Sections 13-15): a manager asked for a single ros2 launch entry point to actually run and watch this demo, rather than the 7-step manual walkthrough that was the only way to run it before. Two launch files now wrap one orchestrator script, with gui, record, block_x/block_y and attach_mode exposed as ordinary, documented ros2 launch arguments — attach_mode deliberately, so the simulated-attachment limitation is visible at the point of use, not only in this document.

Addendum, 2026-10-03 (Sec.16–30): the GUI demo of Sec.15 was finally run and watched in the Gazebo window, on a WSL2 laptop, from a machine with no image and no container. Five problems had to be solved on the way (Docker daemon, missing moveit_py, a frozen simulation caused by DISPLAY, a white Gazebo window, and a camera stuck far away). They are documented step by step, and a new script, run_gui_demo.sh, reduces the whole demo to one command.

This report follows the same structure as the runbooks in Gazebo_to_LeRobot_Pipeline/docs/ and ROS2_to_RLDS_Conversion_OpenVLA/docs/: a step-by-step account pairing what was done with the concrete check that proves it, a reference section for bugs and limitations, and a manual, human-runnable walkthrough at the end.

# Spec vs. implemented, part by part

The specification's own part numbers, what each asked for, and exactly what was delivered against it.

| Spec part | Asked for | Delivered |
| --- | --- | --- |
| Part 1 | A standalone red_block.sdf: 0.05 kg, 4 cm cube, inertia 1.333e-05, mu=1.5, contact kp=1e6/kd=1.0. | Implemented verbatim, then extended: kp/kd were later softened to 5e4/50.0 during the grasp experiment (Sec.8) and a DetachableJoint plugin was added to the same file (Sec.9). |
| Part 2 | Spawn at (0.25, 0.00, 0.05); vary position across episodes (worked examples: x in [0.22,0.28]). | Spawn command and primary pose match exactly. Position variation used x in [0.25,0.31] and y in [-0.08,0.08] instead — see Sec.7, the elbow-up IK branch has no solution below x~0.245 on this arm. |
| Part 3 | Two independent checks: numeric pose via gz model -p, visual via a saved camera frame. | Both implemented; used throughout, not just once (Sec.2, Sec.6). |
| Part 4 | A static plate.sdf (r=0.06, len=0.008), spawned at (0.20, -0.15, 0.044), away from the block. | Implemented verbatim, unchanged throughout the project. |
| Part 5 | A Cartesian IK helper via moveit_py's RobotState.set_from_ik, rejecting the elbow-down branch. | Implemented, with two corrections: the target link is gripper_base, not a generic tip-link name (Sec.3); and moveit_py cannot share a process with a plain rclpy node, forcing an offline/online split (Sec.3-4). |
| Part 6 | An 8-phase scripted sequence; fixed timing constants; grasp height at block_z+0.01. | 10-phase sequence. Timing constants replaced with blocking on the real MoveGroup action result plus closed-loop correction. Grasp height replaced with an empirically measured fingertip offset (Sec.7). |
| Part 7 | A CSV logger; an automatic grasp_held() pass/fail on dz constancy; a lift-peak camera frame. | Implemented, then hardened: the spec's dz-constancy check trivially passes when the block never moves at all. grasp_held() now also requires the block to rise >=5 cm (Sec.6). |
| Part 8 | Record via rosetta's recorder; a first batch of 20 episodes. | Implemented, with the recorder interface differing sharply: episode_recorder_node is a ROS2 lifecycle node needing explicit configure->activate (Sec.10). Batch composition matches spec Sec.8.3's table in structure. |
| Part 9 | Maps the work to the FDT work-order sub-studies. | Out of scope for this report by design. |
| Part 10 | States the limits: simulated grasping is not real grasping; domain gap; scripted motion; dataset size. | Exceeded: rigid-body contact physics could not produce a hold at all after a bounded experiment (Sec.8), so what ships is a simulated attachment, one level cruder than Part 10 assumed. |
| Appendix | An 8-step order of work, flagging step 5 (IK) as the most likely to overrun. | Step 5 did overrun, exactly as flagged. Step 8 (recording) also overran, from environment friction, not defects in the recording logic once fixed. |

The one sentence that matters most: every recorded episode is real arm motion, a real gripper close, and a real camera stream — but the block's carry is a scripted weld, not friction.

# The implementation, step by step

Fifteen steps, in the order they had to happen. Each pairs what was done with the check that proves it, and calls out where a bug was found and fixed along the way.

## 01  Author the block and the plate

Two standalone SDF models, spawned at runtime — nothing added to the world file, everything reversible.

| PROCEDURE models/red_block.sdf and models/plate.sdf, copied from the specification's own Parts 1 and 4 essentially unchanged: block mass 0.05 kg, 0.04 m cube, diagonal inertia 1.333e-05, friction mu=mu2=1.5, contact kp=1e6/kd=1.0; plate a static 0.06 m-radius, 0.008 m-thick cylinder. The block's contact stiffness and a DetachableJoint plugin were added later (Sec.8-9) — this step is the pure-spec baseline. | VERIFY gz sdf -k models/red_block.sdf gz sdf -k models/plate.sdf. Both print nothing (valid) — the spec's own definition of done for step 1, met exactly. |
| --- | --- |

## 02  Spawn headless and verify the pose

Two independent checks, per spec Part 3: numeric pose and a visual camera frame.

| PROCEDURE ros2 run ros_gz_sim create -world empty -file models/plate.sdf \ -name plate -x 0.20 -y -0.15 -z 0.044 ros2 run ros_gz_sim create -world empty -file models/red_block.sdf \ -name red_block -x 0.25 -y 0.0 -z 0.05. scripts/grab_frame.py subscribes to /synth_camera/image/compressed, decodes with cv2.imdecode, saves with cv2.imwrite. | VERIFY gz model --list gz model -m red_block -p. Confirmed the block settles at exactly (0.250, 0.000, 0.020) in a fresh sim. The saved PNG confirmed the block genuinely red and resting on, not inside, the ground plane. |
| --- | --- |

## 03  Cartesian IK helper

The one piece of genuinely new code the spec flagged — and the step where two corrections beyond the spec's sketch were required.

| PROCEDURE scripts/ik_helper.py builds a MoveItPy instance and solves with RobotState.set_from_ik("arm", target, TARGET_LINK, timeout=1.0), rejecting any solution with J3>=0 (elbow-down) and retrying with a random seed. Seed chaining across waypoints keeps a whole sequence on one consistent branch. | VERIFY fk_check() cross-checks every IK solution: FK-of-IK reproduces the requested pose to ~1e-7 m. |
| --- | --- |

Correction beyond the spec: the real target here is gripper_base, a TF frame whose fingertip offset had to be measured (Sec.7) — targeting link6 or a naive fingertip name drives the real fingers underground. The top-down orientation quaternion is also frame-specific and was derived empirically.

## 04  Precompute the waypoints offline

A better solution than the spec anticipated: moveit_py and a plain rclpy node cannot coexist in one process on this stack, so IK is solved once, offline, before the sequence ever imports rclpy.

| PROCEDURE python3 scripts/precompute_ik.py \ --block-x 0.25 --block-y 0.00 --block-z 0.02 \ --tip-offset 0.105 --tip-dx -0.036 --hover 0.08. Solves home/approach/descend/lift/transport/place/retreat in order, chaining each solution as the next seed, and writes waypoints.json. | VERIFY Printed J1/J3 per waypoint; a batch run is only accepted if every waypoint's J1 stays within +/-1.2 rad and J3<0 — the elbow-up, forward-facing branch used for every recorded episode. |
| --- | --- |

Not in the spec, discovered the hard way: constructing MoveItPy in the same process as an explicit rclpy.init()+Node made the MoveItPy constructor hang indefinitely; and separately, letting a MoveItPy instance be garbage-collected tears down its internal rclcpp context and silently breaks any other rclpy activity in that process.

## 05  Script the pick-and-place sequence

scripts/run_pick_and_place.py — plain rclpy, no moveit_py import, consuming waypoints.json.

| PROCEDURE Ten phases (home -> open -> approach -> descend -> grasp -> lift -> transport -> place -> release -> retreat -> home). Each arm move goes through the real MoveGroup action and blocks on its result; move_to_joints_closed_loop re-issues a corrective move (up to 4 times) if the achieved joints differ from target by more than 0.02 rad. | VERIFY Every phase logs "move OK"/"move FAILED" from the action's own error_code, and a pose snapshot is written after every phase — feeding the grasp-proof CSV in Sec.6. |
| --- | --- |

Two more bugs fixed here: (1) trajectory_execution.allowed_start_tolerance at its default 0.01 caused every execution after the first to fail with CONTROL_FAILED — raised to 0.2. (2) The gripper command was originally published once, dropped by the classic ROS2 discovery race; fixed by republishing periodically.

## 06  Prove the grasp numerically

Spec Part 7, implemented as specified — then found to have a real loophole and hardened.

| PROCEDURE A CSV row per phase snapshot: t, phase, bx, by, bz, ex, ey, ez, dz (dz = ee_z - block_z). A lift-peak camera frame is saved for visual corroboration. | VERIFY def grasp_held(rows, tol=0.008, min_rise=0.05): lift = [r for r in rows if r["phase"] in ("lift", "transport")] ref = lift[0]["dz"] constant = all(abs(r["dz"] - ref) < tol for r in lift) rose = max(r["bz"] for r in lift) - descend["bz"] > min_rise return constant and rose. |
| --- | --- |

Loophole found in the spec's own check: the specification's grasp_held() checks only that dz stays constant. A block that never moves at all has a trivially constant dz. The check now also requires the block to rise >=5 cm during lift.

## 07  Where the block actually settles

The finding that explained why early grasp attempts closed on empty air: the spec's nominal spawn coordinates are not where the gripper needs to be.

| PROCEDURE Measured, via TF and gz model -p: fingertip joint origins sit ~0.10 m below gripper_base when pointing down, and the finger pair's meeting point in x is ~0.036 m ahead of gripper_base. precompute_ik.py's --tip-offset 0.105 --tip-dx -0.036 defaults encode these two measurements directly. | VERIFY With the correction applied, the gripper's closing fingers made contact with the block for the first time in this project — the first evidence that geometry, not friction, had been the blocking issue all along. |
| --- | --- |

Practical consequence for Sec.2's position-variation range: the elbow-up IK branch has no solution below x~0.245 once this offset is folded in.

## 08  Attempt a physical friction grasp — bounded experiment, negative result

One controlled variable, three clean-restart runs, as instructed, before concluding a friction grasp was not achievable in this sim in the time available.

| PROCEDURE Block contact softened from kp=1e6/kd=1.0 to kp=5e4/kd=50.0; gripper squeeze target tested at -0.40, -0.55 and -0.70. Grasp geometry held fixed at the Sec.7 offsets. | VERIFY Run; Squeeze target; Gripper stall; Block held?. A; -0.40; -0.334; No. B; -0.55; -0.375; No. C; -0.70; -0.201; No. |
| --- | --- |

Reported as a problem, per the governing instruction: this is the one specification goal — a rigid-body friction grasp — not achieved.

## 09  Simulated attachment — the mechanism actually used

Since a friction grasp was not achievable in the time available, the block is carried by an explicit, clearly-labelled scripted weld instead.

| PROCEDURE A gz-sim-detachable-joint-system plugin in red_block.sdf, welding the block to the robot's link6 on a message to /htgpp/attach and releasing on /htgpp/detach. Every call is logged with the literal string SIMULATED ATTACHMENT. | VERIFY Block z: 0.0200 -> 0.099 (lift) -> 0.152 (transport) -> 0.068 (on the plate), with dz constant through lift and transport. |
| --- | --- |

Gazebo quirk found here: gripper_base is a TF/URDF frame only — after conversion it is lumped into the link6 Gazebo link, so it does not exist as a spawnable-model link name. A DetachableJoint with child_link: gripper_base accepts the attach message and silently does nothing.

## 10  Record twenty episodes

scripts/episode.sh / batch.sh: a per-episode clean-restart driver, matching spec Sec.8.3's batch table.

| PROCEDURE Per episode: kill and relaunch the whole sim stack, spawn plate and block, wait for all three controllers active, launch move_group, configure and activate episode_recorder_node, then run the sequence with --record. Up to 3 attempts per episode. | VERIFY 20/20 episodes ended with run_exit_code=0, placed_on_plate=true, grasp_held=true, and a non-empty .mcap bag. |
| --- | --- |

Three infrastructure bugs found and fixed during this batch: recorder lifecycle (configure->activate needed); stale ros2cli discovery (--no-daemon --spin-time 15 fixed it); cold-start controller races (sequential-respawn fallback).

## 11  Ground-truth batch status

scripts/status.sh — added because the recording batch spanned multiple container restarts (host suspends).

| PROCEDURE Runs from the plain host shell. For each episode it reads grasp_meta.json's own exit-code/verdict fields and checks for a non-empty .mcap bag file. It also compares wall-clock elapsed since container start against PID 1's monotonic uptime. | VERIFY bash scripts/status.sh # PASSED=20 FAILED=0 IN-PROGRESS=0 MISSING=0. |
| --- | --- |

## 12  Convert to a LeRobot-layout dataset, without torch

Not in the spec at all; built here because the rosetta_port/lerobot toolchain needs torch, which this machine could not install reliably.

| PROCEDURE scripts/port_bag.py reads each episode's .mcap bag directly with rosbag2_py and writes the documented LeRobot v3.0 layout with pyarrow (parquet) and ffmpeg (mp4) — no lerobot/torch import anywhere. | VERIFY mycobot_pick_place_train (episodes 1-15): 2670 frames. mycobot_pick_place_heldout (16-20): 927 frames. ffprobe on one clip matches the frame count at 30 fps exactly. |
| --- | --- |

Deferred, not skipped: this has not been loaded through the real LeRobotDataset class — that check needs torch and happens on the GPU machine.

# Addendum, 2026-09-22: a testable launch file

Not from the specification — a direct request from the project manager, quoted in full because it states its own reasoning: « Ce qui serait intéressant est de mettre en place aussi un launch pour qu'on puisse le tester. Si j'ai bien compris tout est fait et validé en backstage automatisé, sans vérification 3D sur gazebo, vu que ta machine ne te le permet pas. Peux-tu t'assurer que le pipeline de lancement sur mon PC soit prêt et que cela fonctionne comme prévu ? » Legitimate, and genuinely not there before Sec.15: this project had sim_full.launch.py and move_group.launch.py, but the demo itself was orchestrated by episode.sh/batch.sh plus the 7-step manual walkthrough, with & backgrounding and until...sleep polling. No single entry point existed.

## 13  IK robustness for a manager-chosen block position

The recording batch only ever needed 9 pre-vetted block positions. A launch-file argument lets a reviewer choose any reachable position and re-run — which meant the single-pose IK path needed the same protection the batch path already had.

| PROCEDURE The branch-consistency retry (seed from a known-good configuration, retry up to 6 times, reject any solution outside the forward-facing elbow-up branch) previously lived only in precompute_ik.py's --poses batch path. Extracted into solve_waypoints_robust() and now shared by the single-pose path too. | VERIFY Explicit-position test at (0.25, 0.00): approach J1=+0.367 descend J3=-2.420 branch_ok=True — the same branch every recorded episode used. Live-detection test (reading the block's actual settled pose): identical result, once the timeout bug below was fixed. |
| --- | --- |

Bug found and fixed: live_block_xyz() raised RuntimeError: could not read live pose of red_block: '' — genuinely empty output, not the documented "prints then hangs" partial output. It runs right after MoveItPy's own ~90 s build; this machine's CPU load at that exact moment was apparently enough to starve the gz model subprocess past its 10 s timeout before it printed anything at all. Fixed by retrying (5x, 25 s each).

## 14  run_demo.py — one orchestrator behind both launch files

Ports scripts/episode.sh's proven bring-up sequence to Python, generalised for an arbitrary block position and an optional recorder, with clean process teardown.

| PROCEDURE Bring up sim_full.launch.py (GUI or headless per argument) -> poll the three controllers active, with the same sequential-respawn fallback episode.sh uses -> spawn plate and block -> wait, then read the block's settled pose -> solve IK against that settled pose (Sec.13) -> bring up move_group -> if record:=true, start and lifecycle-activate the rosetta recorder -> run run_pick_and_place.py -> print the verdict. record:=false (the default) skips the recorder step entirely. | VERIFY Two independent full end-to-end runs (block at (0.25, 0.00), headless), each printing RESULT: motions_ok=True placed_on_plate=True grasp_held=True. Invalid attach_mode tested separately: rejected immediately with exit code 2. |
| --- | --- |

Three bugs found and fixed here, all specific to running this as a launch-file child rather than a bash script (none existed in episode.sh, which was never invoked this way):

- Self-inflicted kill: the stale-process cleanup's "ros2 launch" pattern matched run_demo.py's own parent, since it is now itself launched by a ros2 launch process. It killed its own parent within the first second of every run. Narrowed to the two specific inner launch files that need cleaning up (sim_full.launch.py, move_group.launch.py).

- Timeout crashed instead of retrying: the sh() helper let subprocess.run's TimeoutExpired propagate and crash the whole script on the very first controller poll, which is expected to time out repeatedly. sh() now catches it and returns gracefully, like bash's own timeout command.

- Cleanup didn't actually clean up: sending SIGINT/SIGTERM to the outer ros2 launch sim_full.launch.py subprocess handle does not reliably cascade to Gazebo/move_group — confirmed directly. Backstopped with the same pkill patterns pkill_stale() uses at the start of a run, now also run at the end. See the upstream candidate below.

## 15  Two launch files: a single entry point

One for the manager, with the Gazebo GUI; one for this machine, headless, exiting with a pass/fail code.

| PROCEDURE ros2 launch pick_and_place_demo.launch.py # with Gazebo GUI ros2 launch pick_and_place_no_gui_demo.launch.py # headless, exits pass/fail. | VERIFY ros2 launch pick_and_place_no_gui_demo.launch.py --show-args. Prints all declared arguments with their descriptions, including attach_mode's, which states the limitation directly. |
| --- | --- |

| Argument | Default | Why |
| --- | --- | --- |
| gui | true (demo) / fixed false (no-gui) | The entire point of the GUI variant is seeing it; the entire point of the no-GUI variant is not needing to. |
| record | false | Watching, not data collection — with record:=false, no rosetta install is needed at all. |
| block_x, block_y | 0.25 / 0.00 | Move the object and re-run (Sec.13 made this safe for any reachable point). |
| attach_mode | simulated | Makes the weld visible in the command itself, and validated: any other value is a hard error naming Sec.8-9. |

gui:=true stays open on purpose: after printing the verdict, the GUI variant does not tear anything down — Gazebo and move_group keep running so the final scene stays on screen. Ctrl+C the launch to shut everything down.

Not visually verified: GUI rendering itself was not checked — this machine has no display. The no-GUI variant was verified twice end to end; the GUI variant shares the identical orchestrator and was checked for correct argument-passing and startup only.

Update, 2026-10-03: now visually verified. The GUI variant was run and watched end to end on a WSL2 laptop; on that kind of machine it needs the fixes of Sec.24–26, all wrapped in run_gui_demo.sh (Sec.27).

# Addendum, 2026-10-03: watching the GUI demo on a WSL2 laptop, from an empty machine

Sec.15 shipped pick_and_place_demo.launch.py without anyone having watched it: the machine it was written on had no display. On 2026-10-03 it was run and watched end to end in the Gazebo GUI on a Windows laptop (Intel Iris Xe graphics) running Ubuntu 22.04.3 under WSL2 (kernel 6.18.40.1-microsoft-standard-WSL2), with Docker Engine 24.0.5 installed inside WSL itself (no Docker Desktop). Starting point: the Docker daemon was not running, and neither the gazebo_to_lerobot image nor the container existed. Five separate problems stood between the one-line command in Sec.15 and a visible pick-and-place. Each step below gives the exact commands, which terminal to type them in, what you should see, and the symptom of each problem as it appears on screen, so you can recognise it. Every command was actually run; every output quoted is real.

### Short version, once the first-time setup (Sec.17–Sec.24) is done

# In a normal Ubuntu (WSL) terminal, after every Windows reboot or `wsl --shutdown`: sudo sh -c 'nohup dockerd > /tmp/dockerd.log 2>&1 &' docker start gazebo_to_lerobot # Then, every time you want to watch the demo (same terminal): docker exec -it gazebo_to_lerobot /workspace/htgpp/run_gui_demo.sh # another block position: docker exec -it gazebo_to_lerobot /workspace/htgpp/run_gui_demo.sh block_x:=0.28 block_y:=0.05 # Ctrl+C in that terminal stops everything.

## 16  Where the command lives, and which terminal it runs in

The first question anyone asks after reading Sec.15: where is pick_and_place_demo.launch.py, and from which terminal do I type ros2 launch pick_and_place_demo.launch.py?

The file is launch/pick_and_place_demo.launch.py in this project, i.e. ~/tomislav/robotics/mycobot_320pi_R6A_tomislav_02_10_2026/Headless_Task-Grounded_Pick-and-Place_in_Gazebo/launch/pick_and_place_demo.launch.py on the laptop (adapt the prefix to wherever you cloned the repository). Its neighbour pick_and_place_no_gui_demo.launch.py is the headless variant.

ros2 launch pick_and_place_demo.launch.py does not work as typed from an arbitrary directory. This project is not a ROS package: it has no package.xml and is never built with colcon, so ros2 launch cannot find the file by name. It accepts a file path instead: either the full path, or the bare file name when your current directory is the launch/ directory itself.

It does not work from your normal Ubuntu terminal either. The launch file starts scripts/run_demo.py, which needs ROS2 Jazzy, Gazebo Harmonic, MoveIt2, moveit_py, and the gazebo_to_lerobot_bringup and mycobot_moveit_config packages. All of these exist only inside the gazebo_to_lerobot Docker container. You will use two kinds of terminal, and the prompt tells you which one you are in:

| Terminal | Prompt looks like | Used for |
| --- | --- | --- |
| Host terminal: a normal Ubuntu (WSL) terminal on the laptop | tilic@LAPTOP-7BFDQAO6:~$ | Starting Docker, docker build, docker cp, docker exec, copying files |
| Container shell: opened with docker exec -it gazebo_to_lerobot bash | root@c76def923a51:/workspace# | ros2 …, gz …, colcon, and the launch file itself |

Inside the container, the launch files are at /workspace/htgpp/launch/ once Sec.23 has copied them there. The exact form of the Sec.15 command then works:

# container shell source /opt/ros/jazzy/setup.bash source /workspace/install/setup.bash cd /workspace/htgpp/launch ros2 launch pick_and_place_demo.launch.py

On a WSL2 laptop, that command alone is not enough. The simulation freezes unless the shell was opened with -e DISPLAY=:0 (Sec.24), and the Gazebo window it opens stays white (Sec.25). Sec.27 wraps everything into one command that avoids both.

## 17  Start the Docker daemon

On this kind of WSL2 install, the Docker daemon does not start by itself, and the usual commands to start it do not work.

Symptom. Any docker command answers:

Cannot connect to the Docker daemon at unix:///var/run/docker.sock. Is the docker daemon running?

Why the usual fixes fail. WSL here runs without systemd (ps -p 1 -o comm= prints init(Ubuntu)), so sudo systemctl start docker answers System has not been booted with systemd as init system (PID 1). Can't operate., and there is no /etc/init.d/docker script, so sudo service docker start does nothing either. The daemon must be started by hand.

Procedure (host terminal)

groups # must list 'docker'; otherwise: sudo usermod -aG docker $USER, then reopen WSL sudo sh -c 'nohup dockerd > /tmp/dockerd.log 2>&1 &' # asks for your Linux password once; the daemon then keeps running in the background

Verify

docker ps -a --format '{{.Names}}\t{{.Status}}'

Any listing, even an empty one, means the daemon answers. If docker still cannot connect, read /tmp/dockerd.log.

Repeat this after every Windows reboot or wsl --shutdown. The daemon dies with the WSL VM. This was observed on 2026-10-03: the daemon started the day before was gone the next morning.

## 18  Build the image, if the container does not exist

On 2026-10-03 the listing in Sec.17 showed only five unrelated containers from 2022: no gazebo_to_lerobot, and docker images showed no gazebo_to_lerobot image either.

If gazebo_to_lerobot is in the Sec.17 listing, skip to Sec.22: the image, the container and its workspace already exist. Otherwise build the image from the sibling project Gazebo_to_LeRobot_Pipeline, as its own runbook (step 01) describes.

Procedure (host terminal)

cd ~/tomislav/robotics/mycobot_320pi_R6A_tomislav_02_10_2026/Gazebo_to_LeRobot_Pipeline docker build -t gazebo_to_lerobot:jazzy-harmonic -f docker/Dockerfile docker

This downloads the ros:jazzy-ros-base base image and installs ROS desktop, Gazebo Harmonic, ros2_control and MoveIt2 into it. The result is 4.77 GB; expect tens of minutes, depending on your connection.

Verify

docker images gazebo_to_lerobot # gazebo_to_lerobot jazzy-harmonic ... 4.77GB

The build log ends with Successfully tagged gazebo_to_lerobot:jazzy-harmonic.

## 19  Start the persistent container

One long-lived container, with the WSLg display sockets and the /dev/dxg GPU device passed in.

Procedure (host terminal, still in Gazebo_to_LeRobot_Pipeline)

bash docker/run.sh

run.sh re-runs docker build (instant now, every layer is cached), then docker run -d --name gazebo_to_lerobot …, mounting /tmp/.X11-unix, /mnt/wslg and /usr/lib/wsl, and bind-mounting Gazebo_to_LeRobot_Pipeline/src to /workspace/src. It refuses to run if a container with that name already exists (remove it first with docker rm -f gazebo_to_lerobot only if you really want a clean start: that also deletes Sec.22's install).

Verify

docker ps --format '{{.Names}}\t{{.Status}}' # gazebo_to_lerobot Up ...

run.sh copies your host DISPLAY into the container (-e DISPLAY="${DISPLAY:-:0}"). On this laptop the host DISPLAY is 172.24.112.1:0, which breaks the simulation (Sec.24). That is why every later docker exec in this addendum passes -e DISPLAY=:0 explicitly. run.sh itself was not modified.

## 20  Copy the robot description into the workspace

Gazebo_to_LeRobot_Pipeline keeps only three hand-modified files under overrides/, not the 135 MB mesh tree. The two robot packages must be copied in from the main repository, then the overrides applied on top (pipeline runbook, step 03).

Procedure (host terminal, in Gazebo_to_LeRobot_Pipeline)

mkdir -p src/mycobot_R6A cp -r ../mycobot_description ../mycobot_gateway src/mycobot_R6A/ cp -r overrides/mycobot_description/* src/mycobot_R6A/mycobot_description/ cp -r overrides/mycobot_gateway/* src/mycobot_R6A/mycobot_gateway/

Verify

for f in $(cd overrides && find . -type f); do diff -q overrides/$f src/mycobot_R6A/$f && echo "OK $f"; done # OK ./mycobot_gateway/launch/mycobot_teleop.launch.py # OK ./mycobot_description/config/controller.yaml # OK ./mycobot_description/urdf/320_pi/mycobot_pro_320_pi_gazebo.urdf grep -c "<always_on>false" src/mycobot_R6A/mycobot_description/urdf/320_pi/mycobot_pro_320_pi_gazebo.urdf # 3

src/mycobot_R6A/ is gitignored in Gazebo_to_LeRobot_Pipeline, so this copy leaves git status clean. Because run.sh bind-mounts src/, the container sees the copy immediately.

## 21  Build the ROS workspace inside the container

Import the five rosetta repositories, install their system dependencies, and build all nine packages.

Procedure (host terminal; the command runs inside the container)

docker exec gazebo_to_lerobot bash -c 'source /opt/ros/jazzy/setup.bash && cd /workspace \ && vcs import src < src/rosetta.repos \ && apt-get update \ && rosdep install --from-paths src --ignore-src -r -y \ && colcon build --symlink-install'

A few minutes in total; colcon build itself took 21 s. rosdep prints an ERROR that is expected and harmless, because -r makes it continue:

ERROR: the following packages/stacks could not have their rosdep keys resolved to system dependencies: lerobot_rosetta: Cannot locate rosdep definition for [ament_python] ... (same line for rosetta, lerobot_teleoperator_rosetta, gazebo_to_lerobot_bringup, lerobot_robot_rosetta) Continuing to install resolvable dependencies...

Verify

# last lines of the build: Summary: 9 packages finished [21.3s] docker exec gazebo_to_lerobot bash -c 'source /opt/ros/jazzy/setup.bash; source /workspace/install/setup.bash; ros2 pkg list ¦ grep -E "mycobot¦gazebo_to_lerobot¦rosetta"' # gazebo_to_lerobot_bringup, lerobot_robot_rosetta, lerobot_rosetta, lerobot_teleoperator_rosetta, # mycobot_description, mycobot_gateway, mycobot_moveit_config, rosetta, rosetta_interfaces

## 22  Install moveit_py, which the image does not contain

The Dockerfile installs ros-jazzy-moveit, which does not include MoveIt's Python bindings. precompute_ik.py (Sec.3–4) cannot run without them. The container this project was developed in must have had them added by hand; that step was never written down until now.

Procedure (host terminal)

docker exec gazebo_to_lerobot bash -c 'apt-get install -y ros-jazzy-moveit-py'

Installed version on 2026-10-03: ros-jazzy-moveit-py 2.12.4-1noble.20260903.100911.

Verify

docker exec gazebo_to_lerobot bash -c 'source /opt/ros/jazzy/setup.bash; python3 -c "import moveit; print(\"moveit_py OK\")"' # moveit_py OK

Misleading symptom if you skip this. The demo stops about one second after solving IK for the settled block pose with: FATAL: no reachable elbow-up IK solution for this block position (0.25, 0.0) -- try a position closer to (0.25, 0.00). The position is fine. The real error, visible only by running python3 precompute_ik.py by hand inside the container, is ModuleNotFoundError: No module named 'moveit'. run_demo.py prints only the standard output of precompute_ik.py, so the traceback (on standard error) is lost and every failure is reported as unreachable. Not fixed in run_demo.py; documented here.

The install lives in the container, not the image. Stopping and restarting the container keeps it. docker rm followed by run.sh produces a container without moveit_py again: redo this step.

## 23  Copy this project into the container

The container never sees this project's directory: only Gazebo_to_LeRobot_Pipeline/src is mounted. Everything is copied to /workspace/htgpp/.

Procedure (host terminal, in Headless_Task-Grounded_Pick-and-Place_in_Gazebo)

cd ~/tomislav/robotics/mycobot_320pi_R6A_tomislav_02_10_2026/Headless_Task-Grounded_Pick-and-Place_in_Gazebo docker exec gazebo_to_lerobot mkdir -p /workspace/htgpp for d in launch scripts models contracts; do docker cp $d gazebo_to_lerobot:/workspace/htgpp/; done # the scripts are ALSO needed flat in /workspace/htgpp/ itself: for f in scripts/*.py scripts/*.sh; do docker cp $f gazebo_to_lerobot:/workspace/htgpp/; done

Why both copies of the scripts. run_demo.py calls /workspace/htgpp/precompute_ik.py, not /workspace/htgpp/scripts/precompute_ik.py: the container was always deployed with every script flattened into the project root (Sec.14's comment in the launch file). The launch file itself finds run_demo.py in either place. The second loop also installs run_gui_demo.sh (Sec.27) at /workspace/htgpp/run_gui_demo.sh.

Verify

docker exec gazebo_to_lerobot ls /workspace/htgpp # batch.sh contracts episode.sh grab_frame.py ik_helper.py launch models port_bag.py # precompute_ik.py run_demo.py run_gui_demo.sh run_pick_and_place.py scripts status.sh test_ik.py

Redo this step after any change to the project's scripts, launch files or models on the host: the container holds copies, not links.

## 24  Headless check, and the frozen simulation (DISPLAY)

Run the headless variant first: it is the one Sec.15 verified, it exits with a pass/fail code, and it isolates simulation problems from display problems.

Procedure (host terminal)

docker exec -e DISPLAY=:0 gazebo_to_lerobot bash -c 'source /opt/ros/jazzy/setup.bash; source /workspace/install/setup.bash ros2 launch /workspace/htgpp/launch/pick_and_place_no_gui_demo.launch.py; echo "exit code: $?"'

Verify

About 4 minutes later: RESULT: motions_ok=True placed_on_plate=True grasp_held=True. On 2026-10-03 the controllers became active 16–19 s after launching sim.

Symptom without -e DISPLAY=:0: the simulation is frozen. The log shows controllers not all active yet -- respawning the missing ones sequentially, then about 4.5 minutes later FATAL: controllers never reached 'active' -- see sim.log. In /workspace/htgpp/demo/sim.log the controller manager logs Activating controllers: [ joint_state_broadcaster ] again every 60 s and the spawners log Failed getting a result from calling /controller_manager/switch_controller; mycobot_controller is never even loaded. The simulation clock is not advancing: gz topic -e -t /world/empty/clock -n 1 (inside the container) times out although the topic has a publisher.

Cause. On this laptop the host's DISPLAY is 172.24.112.1:0 (check with echo $DISPLAY in a host terminal), the address of an external X server (VcXsrv style) that nothing answers. run.sh passes it into the container. The camera sensor in the simulated world needs an OpenGL context; opening it blocks on that dead address, and since sensors are updated inside the simulation step, the whole simulation stops. The same hang is visible directly: glxinfo -B in the container never returns. WSLg's own display, :0 (socket /tmp/.X11-unix/X0), works:

docker exec -e DISPLAY=:0 gazebo_to_lerobot glxinfo -B ¦ grep renderer # OpenGL renderer string: D3D12 (Intel(R) Iris(R) Xe Graphics)

Fix. Pass -e DISPLAY=:0 to every docker exec that runs the simulation, including docker exec -it -e DISPLAY=:0 gazebo_to_lerobot bash when you open a container shell. run_gui_demo.sh (Sec.27) sets it itself.

## 25  The white Gazebo window: use software OpenGL

With DISPLAY=:0, pick_and_place_demo.launch.py runs and the Gazebo window opens, but its 3D view stays completely white for the whole run, while the log shows the arm doing the full sequence.

Cause. Through WSLg, the Intel GPU is reached via Mesa's D3D12 translation layer, which offers only OpenGL 4.1, without compute shaders. ~/.gz/rendering/ogre2.log in the container shows GL_RENDERER = D3D12 (Intel(R) Iris(R) Xe Graphics), GL_VERSION = 4.1 (Core Profile) and Compute programs: no. Gazebo's default ogre2 engine draws a white viewport with that. The older engine (gz sim -g --render-engine ogre) was tried and was white too, so the problem is the D3D12 path, not the engine.

Fix. Make Mesa render on the CPU (llvmpipe), which offers OpenGL 4.5. Three environment variables; the third is set to an empty value to cancel the MESA_LOADER_DRIVER_OVERRIDE=d3d12 that run.sh puts into the container:

LIBGL_ALWAYS_SOFTWARE=1 GALLIUM_DRIVER=llvmpipe MESA_LOADER_DRIVER_OVERRIDE=

Verify

docker exec -e DISPLAY=:0 -e LIBGL_ALWAYS_SOFTWARE=1 -e GALLIUM_DRIVER=llvmpipe -e MESA_LOADER_DRIVER_OVERRIDE= \ gazebo_to_lerobot glxinfo -B ¦ grep -E 'renderer string¦core profile version' # OpenGL renderer string: llvmpipe (LLVM 20.1.2, 256 bits) # OpenGL core profile version string: 4.5 (Core Profile) Mesa 25.2.8-0ubuntu0.24.04.4

First test: a second window. A second GUI started with these variables and connected to the already-running simulation showed the scene correctly:

docker exec -e DISPLAY=:0 -e LIBGL_ALWAYS_SOFTWARE=1 -e GALLIUM_DRIVER=llvmpipe -e MESA_LOADER_DRIVER_OVERRIDE= \ gazebo_to_lerobot bash -c 'source /opt/ros/jazzy/setup.bash; gz sim -g --force-version 8'

Why this two-window approach was dropped. It needs a second terminal opened at the right moment. controllers active scrolls past within 20 s of the start; by the time the second window was open and zoomed in, the arm had finished and only a static scene was left. Sec.27 instead gives the variables to the launch itself, so the one window it opens renders from the start. Measured cost: none on startup (controllers active in 20 s, as with the GPU path); motion in the window is a little less fluid.

## 26  The tiny view, and the camera that zooms out by itself

Once the window renders, the arm is very small in the middle of the view. Two separate causes, and one of them looks like Gazebo zooming out on its own.

Cause 1: the robot's bounding box includes the camera rig. The four scene cameras (front, left, right, top) are links of the mycobot_320 model, so its bounding box spans the whole rig. Move to on mycobot_320 in the Entity Tree flies to the arm and then frames that whole box, which zooms back out. This is noted in the comments of sim_full.launch.py.

Cause 2: Follow mode. If Follow was ever chosen for mycobot_320, the GUI camera stays locked to it at an offset of 3 m back and 2 m up, and pulls itself back there after every zoom, even after Move to red_block. Checked inside the container:

gz topic -e -t /gui/currently_tracked -n 1 # track_mode: FOLLOW # follow_target { name: "mycobot_320" } # follow_offset { x: -3 z: 2 }

Fix by hand (in the Gazebo window)

- Right-click an empty part of the 3D view and choose Unfollow (also available from the right-click menu of the entity in the Entity Tree).

- Then right-click red_block (or plate) in the Entity Tree and choose Move to. Do not use Move to or Follow on mycobot_320.

Fix by command (container shell, opened with -e DISPLAY=:0)

# stop following gz service -s /gui/follow --reqtype gz.msgs.StringMsg --reptype gz.msgs.Boolean --timeout 5000 --req 'data: ""' # place the camera at (0.65, -0.55, 0.45) m, looking at the work area around (0.18, -0.06, 0.05) m gz service -s /gui/move_to/pose --reqtype gz.msgs.GUICamera --reptype gz.msgs.Boolean --timeout 5000 \ --req 'pose: {position: {x: 0.65, y: -0.55, z: 0.45}, orientation: {w: 0.37847, x: -0.24197, y: 0.10319, z: 0.88745}}' # each prints: data: true

The orientation is the quaternion for roll 0, pitch 0.532 rad (looking down), yaw 2.335 rad (towards the work area). After the two commands, gz topic -e -t /gui/currently_tracked -n 1 prints nothing, meaning no tracking, and the view stays close. run_gui_demo.sh (Sec.27) sends the second command automatically.

## 27  run_gui_demo.sh: the whole GUI demo in one command

New file scripts/run_gui_demo.sh. It wraps pick_and_place_demo.launch.py with the three fixes above, so the person watching types one command in one terminal and sees the arm move from the beginning, close up.

What it does, in order:

- Sets DISPLAY=:0 (Sec.24) and the three software-OpenGL variables (Sec.25). The launch, run_demo.py, sim_full.launch.py and Gazebo's GUI all inherit them, so the single window that opens renders correctly.

- Sources /opt/ros/jazzy/setup.bash and /workspace/install/setup.bash, so you do not type them.

- Starts a background helper that waits until the GUI offers the /gui/move_to/pose service, waits 8 s more for the scene to load, then sends the Sec.26 camera command and prints [run_gui_demo] GUI camera framed on the arm.

- Runs ros2 launch pick_and_place_demo.launch.py from /workspace/htgpp/launch, passing on any extra arguments (block_x:=…, block_y:=…, record:=…).

- On exit (Ctrl+C or end of the launch), stops the helper and kills any move_group or gz sim still running. On 2026-10-03 a Ctrl+C in an interactive terminal left a move_group behind, which would make the next run fail.

Install (host terminal, in Headless_Task-Grounded_Pick-and-Place_in_Gazebo)

docker cp scripts/run_gui_demo.sh gazebo_to_lerobot:/workspace/htgpp/run_gui_demo.sh # (already done if you ran Sec.23's second loop after this file existed)

Run (host terminal)

docker exec -it gazebo_to_lerobot /workspace/htgpp/run_gui_demo.sh docker exec -it gazebo_to_lerobot /workspace/htgpp/run_gui_demo.sh block_x:=0.28 block_y:=0.05

No -e DISPLAY=:0 and no source lines are needed here: the script sets them itself. Keep -it, so that Ctrl+C reaches the launch.

Verify: the run of 2026-10-03, minute by minute

| Time | Terminal line | What you see in the Gazebo window |
| --- | --- | --- |
| 04:25:27 | gui=True record=False block=(0.25,0.0) attach_mode=simulated | Nothing yet |
| 04:25:31 | launching sim (GUI)... | The Gazebo window opens, rendered (ground grid, arm, camera rig) |
| a few s later | [run_gui_demo] GUI camera framed on the arm | The view jumps close to the arm |
| 04:25:51 | controllers active. Spawning plate and block... | The plate and the red cube appear |
| 04:26:12 | block settled at (0.25, 0.0, 0.02049) | The cube rests on the ground |
| 04:26:12 | solving IK for the settled block pose ... | Static scene (about 35 s) |
| 04:26:48 | starting move_group... | Static scene (about 60 s) |
| 04:27:49 | running the pick-and-place sequence... | The arm moves: approach, descend, close, lift, transport, place, release, retreat, home |
| 04:30:00 | RESULT: motions_ok=True placed_on_plate=True grasp_held=True | Cube on the plate, arm back at home |

Total: about 4.5 minutes, the arm moving for the last 2. Watched and confirmed on 2026-10-03: the window rendered, the view was close from the start, and the arm was seen carrying the red cube to the plate.

The window stays open on purpose after RESULT, on the final scene (Sec.15). Press Ctrl+C in the terminal to stop everything; run the same command again to watch another run.

## 28  Moving around in the Gazebo window (including with a trackball)

Gazebo Harmonic's basic view controls. They move only your point of view, never the simulation.

| Action | Mouse with a wheel | Trackball without a wheel |
| --- | --- | --- |
| Zoom | Scroll wheel (zooms towards the point under the cursor) | Hold the right button and roll the ball forward (in) or back (out) |
| Pan (slide the view) | Hold the left button and drag | Hold the left button and roll |
| Orbit (rotate around) | Hold the middle button (press the wheel) and drag | Shift + left button, and roll |
| Select an object | Left click | Left click |
| Context menu | Short right click | Short right click (only press-and-drag zooms) |

- Put the cursor on the red cube before zooming: zoom heads towards the point under the cursor.

- Move to red_block or plate (right click in the Entity Tree) frames them closely. Avoid Move to and Follow on mycobot_320 (Sec.26).

- Bottom left, ▶ / ⏸ plays or pauses the simulation: do not leave it paused, or the demo stalls. Bottom right, RTF is the real-time factor; well below 100% is normal on this laptop.

- Many trackballs (Logitech Ergo/M575, Kensington Expert/Orbit and others) can emulate a scroll wheel (scroll ring, or hold a button + roll) through their Windows software (Logi Options+, KensingtonWorks).

Do not use the Translate (T) and Rotate (R) tools of the top toolbar: they move objects in the simulation itself, not the camera. Dragging the cube or the arm spoils the run. Press Esc to return to selection mode.

## 29  Reading the result in the terminal

What the end of the log means, taken from the run you can reproduce (block at (0.25, 0.00)).

[pick_and_place]: [place] correction 1: max_err=0.0057 rad [pick_and_place]: SIMULATED ATTACHMENT: releasing block from link6 [pick_and_place]: [release] gripper reached -0.045 (target 0.0) [pick_and_place]: [retreat] move OK [pick_and_place]: [home_final] move OK All motions succeeded: True Block carried (dz constant AND block rose through lift+transport): True Grasp held (dz constant through lift+transport): True Block final pose: (0.19744, -0.151968, 0.068) placed on plate: True RESULT: motions_ok=True placed_on_plate=True grasp_held=True

- move OK: the MoveGroup action reported success for that phase. correction N: max_err=… lines are the closed-loop fine-tuning of Sec.5, not errors.

- dz is the gripper height minus the cube height. Constant through lift and transport while the cube rises means it was really carried (Sec.6).

- Final pose (0.197, −0.152, 0.068): about 3 mm from the plate centre (0.20, −0.15). The height checks out too: 0.068 m = plate top (0.044 + 0.004 = 0.048 m) + half the 4 cm cube (0.020 m), so the cube rests on the plate.

- SIMULATED ATTACHMENT: the cube is carried by a scripted Gazebo weld closed when the gripper closes and opened when it opens (Sec.8–9), not by friction.

- All three of motions_ok, placed_on_plate and grasp_held must be True for a pass. The same values are in /workspace/htgpp/demo/grasp_meta.json.

## 30  Stopping, re-running, and leftover processes

A clean stop matters: a leftover move_group or Gazebo from the previous run makes the next one fail.

Stop: Ctrl+C in the terminal running run_gui_demo.sh (or the launch). Check from a host terminal that nothing is left:

docker exec gazebo_to_lerobot pgrep -af "gz sim¦move_group¦ros2 launch" # nothing printed = clean (lines marked <defunct> are dead zombies, harmless)

If something is left, for example the ros2 launch mycobot_moveit_config move_group.launch.py observed on 2026-10-03 after a Ctrl+C on the plain launch file:

docker exec gazebo_to_lerobot pkill -INT -f move_group docker exec gazebo_to_lerobot pkill -INT -f "gz sim" # a few seconds later, if still there: docker exec gazebo_to_lerobot pkill -9 -f move_group docker exec gazebo_to_lerobot pkill -9 -f "gz sim"

Do not wrap pkill -f in bash -c. docker exec gazebo_to_lerobot bash -c 'pkill -f move_group; …' kills its own shell: the pattern appears in that shell's own command line, so pkill matches it, and the command ends with exit code 137 or 143 without finishing. This happened four times on 2026-10-03. Call pkill directly through docker exec, as above.

Re-run: run the same docker exec -it gazebo_to_lerobot /workspace/htgpp/run_gui_demo.sh … command again. Each run restarts the simulation from scratch.

End of the session: docker stop gazebo_to_lerobot (optional; the container keeps moveit_py and the built workspace). Next time, start with the short version at the top of this addendum.

# Troubleshooting summary

| What you see | Cause | Fix |
| --- | --- | --- |
| Cannot connect to the Docker daemon | Daemon not started (no systemd in WSL) | Sec.17 |
| docker ps -a has no gazebo_to_lerobot | Image and container never built on this machine | Sec.18–Sec.23 |
| controllers not all active yet, then FATAL: controllers never reached 'active' | Host DISPLAY=172.24.112.1:0 passed into the container freezes the simulation | -e DISPLAY=:0 (Sec.24) |
| FATAL: no reachable elbow-up IK solution about 1 s after solving IK, at a reachable position | moveit_py not installed; real error hidden | Sec.22 |
| Gazebo window opens but stays white | D3D12 GPU path: OpenGL 4.1, no compute shaders | Software OpenGL (Sec.25), built into run_gui_demo.sh |
| Arm tiny; view zooms out again after every zoom-in | Follow on mycobot_320 (offset 3 m back, 2 m up); bounding box includes the camera rig | Unfollow, Move to red_block (Sec.26) |
| Second viewer window opens too late: arm already static | Two-terminal approach | run_gui_demo.sh (Sec.27) |
| No scroll wheel | Trackball | Right-button drag zooms (Sec.28) |
| Next run fails right away after a Ctrl+C | Leftover move_group / gz sim | Sec.30 |
| docker exec … bash -c 'pkill -f …' ends with exit 137/143 | pkill -f matched its own shell | Call pkill without bash -c (Sec.30) |

### What was not changed

To keep this addendum a faithful record, the shared files were left as they were; the fixes live in the commands above and in the new run_gui_demo.sh. Three permanent fixes are proposed, not applied:

- Gazebo_to_LeRobot_Pipeline/docker/Dockerfile: add ros-jazzy-moveit-py, which would make Sec.22 unnecessary.

- Gazebo_to_LeRobot_Pipeline/docker/run.sh: pass DISPLAY=:0 instead of the host's DISPLAY on WSLg, which would make Sec.24's -e DISPLAY=:0 unnecessary.

- scripts/run_demo.py: print the standard error of precompute_ik.py when it fails, so a missing module is no longer reported as an unreachable position (Sec.22).

# Verification ladder

Each rung strictly depends on the one below it. Level 7 is a later addition (Sec.13-15) the specification never asked for: a single testable entry point.

Level 0 — PASSED
Scene is physically valid
Block and plate spawn, settle, and are visible and correctly coloured in a saved camera frame (Sec.2).

Level 1 — PASSED
Arm reaches commanded Cartesian points
IK solves on the elbow-up branch with FK-of-IK error ~1e-7 m; the real MoveGroup action reports success for every phase (Sec.3-5).

Level 2 — STOPPED HERE
Rigid-body friction grasp holds the block
Not achieved after a bounded, three-run experiment (Sec.8). This is the one specification goal genuinely not met.

Level 3 — PASSED, BY SUBSTITUTION
The block is carried and placed, verifiably
Via the simulated DetachableJoint attachment (Sec.9) — scripted attachment, not physics, labelled as such everywhere it appears.

Level 4 — PASSED
20 episodes recorded with a truthful instruction
Every episode's bag exists, is non-empty, and its own grasp_meta.json confirms success (Sec.10-11).

Level 5 — PASSED
Episodes converted to the documented dataset layout
port_bag.py, without torch (Sec.12).

Level 6 — NOT ATTEMPTED HERE
Loaded and validated through the real LeRobotDataset class
Needs torch; deferred to the GPU machine by design (Sec.12).

Level 7 — PASSED
Single ros2 launch entry point, any reachable block position
Verified twice end to end via pick_and_place_no_gui_demo.launch.py (Sec.13-15): RESULT motions_ok=True placed_on_plate=True grasp_held=True both times, clean process teardown. GUI rendering not visually checked.

# Bugs fixed — session-specific

Symptoms of this exact environment, not defects in any upstream project.

| Symptom | Root cause | Fix |
| --- | --- | --- |
| Every MoveGroup execution after the first fails with CONTROL_FAILED | allowed_start_tolerance default (0.01) too tight | Raised to 0.2 |
| Gripper command silently has no effect | One-shot publish dropped by the discovery race | Republish every 0.5 s |
| Gripper never reports "reached target" while genuinely holding an object | A held object stops the fingers short of fully-closed | Accept a >=2 s stall below -0.1 rad as success |
| Grasp attempts closed on empty air next to the block | Nominal spawn coordinates are not where the fingers meet | Measured offsets via TF, folded into precompute_ik.py |
| base->gripper_base TF disappears after ~1.5 h of sim uptime | Correlates with memory pressure | Full stack restart before every episode |
| Container dies mid-run with multi-hour gaps in log timestamps | Host suspend/resume (WSL2 pauses the VM's clock) | Bounded-timeout liveness checks, never a marker file alone |
| pip install lerobot==0.6.0 pulls a full CUDA/triton chain and is killed | No --index-url restriction on transitive deps | Wrote the dataset directly with pyarrow+ffmpeg (Sec.12) |
| Three parallel controller spawners intermittently time out on cold start | Contention for the controller manager's lock | Sequential one-at-a-time respawn fallback |
| Demo orchestrator (Sec.14) killed its own parent within the first second | Stale-process cleanup's "ros2 launch" pattern matched its own parent | Narrowed to the specific inner launch files needing cleanup |
| Demo orchestrator crashed on the very first, expected-to-time-out poll | subprocess.run's TimeoutExpired left to propagate | sh() helper now returns a failed result, like bash's timeout |
| live_block_xyz() (Sec.13) failed with genuinely empty gz model output | 10 s timeout too tight right after MoveItPy's ~90 s build | Retry up to 5x at 25 s each |

# Bugs worth reporting upstream

Four defects that reproduce independent of this VM. Filing the actual issues has not been done and needs a separate explicit go-ahead.

MoveItPy cannot share a process with a plain rclpy node [Not yet filed] Candidate repo: moveit/moveit2 (moveit_py) SYMPTOM: Constructing moveit.planning.MoveItPy in a process where rclpy.init() has already been called makes the MoveItPy constructor hang indefinitely. Separately, letting a constructed MoveItPy instance be garbage-collected tears down an internal rclcpp executor/context that a separately created rclpy Node in the same process was depending on. FIX USED HERE: Architectural split: precompute_ik.py imports only moveit_py and exits; run_pick_and_place.py imports only rclpy. The two processes never coexist. WHY IT MATTERS: Nothing in moveit_py's public API documents this. Both failure modes are silent.

ros2 lifecycle/ros2 action CLI report a running, configured node as absent [Not yet filed] Candidate repo: ros2/ros2cli SYMPTOM: ros2 lifecycle set /episode_recorder configure and ros2 action list both reported the node/action as not found, while ros2 node list in the same shell showed it present. The daemon-backed default discovery appears to serve a stale cache. FIX USED HERE: ros2 lifecycle set --no-daemon --spin-time 15 ... forcing fresh discovery resolved it every time. WHY IT MATTERS: This produced a hard, unexplained failure on a node that was demonstrably healthy.

gz-sim-detachable-joint-system silently no-ops on a non-model child_link [Not yet filed] Candidate repo: gazebosim/gz-sim SYMPTOM: Setting <child_link>gripper_base</child_link> on a DetachableJoint plugin produces no error and no warning. The attach topic accepts the message; nothing attaches; the block simply does not move on lift. FIX USED HERE: Retargeted child_link to link6, the real Gazebo link. WHY IT MATTERS: This cost a full debugging cycle that looked identical to the earlier, genuine friction-grasp failure.

An externally-delivered signal to a ros2 launch process does not reliably shut down what it manages [Not yet filed] Candidate repo: ros2/launch or ros2/launch_ros SYMPTOM: Sending SIGINT then SIGTERM/SIGKILL to a Popen handle for "ros2 launch ... sim_full.launch.py" (started as a subprocess of the Sec.14 orchestrator, itself running as a different launch file's ExecuteProcess) reported the process as exited cleanly — but Gazebo and move_group were still fully alive afterward, confirmed via pgrep. Interactive Ctrl+C on a foreground ros2 launch tears everything down correctly; a signal delivered programmatically to the process object apparently does not reach the same internal shutdown path. FIX USED HERE: Backstopped with an explicit pkill against the specific process patterns after the signal-based attempt. WHY IT MATTERS: Any orchestration that nests a ros2 launch invocation inside another script or launch file can leak Gazebo/node processes silently unless the caller independently tracks and kills what it started.

# What this does not prove

Restating spec Part 10 in full, and adding what changed.

- The carried block is not a physical grasp of any kind. Even rigid-body contact could not be made to hold in the time available (Sec.8).

- Sim-trained data carries a visual domain gap, unchanged from the spec.

- Scripted motion is not demonstration quality, unchanged from the spec.

- Twenty episodes is not a training set, unchanged from the spec.

- Not yet loaded through the real LeRobotDataset class (new, Sec.12).

What this does prove, precisely: the pipeline can produce structurally sound, self-verifying, task-grounded episodes with a truthful instruction, entirely headless, on hardware with no GPU at all — and now, a single testable entry point for it.

# Manual execution, start to finish

Every command a person would type, in order, to reproduce one recorded episode by hand.

# 1. Bring up the sim stack and wait for controllers ros2 launch gazebo_to_lerobot_bringup sim_full.launch.py & until ros2 control list_controllers ¦ grep -q "mycobot_controller.*active"; do sleep 5; done # 2. Spawn the scene ros2 run ros_gz_sim create -world empty -file models/plate.sdf -name plate -x 0.20 -y -0.15 -z 0.044 ros2 run ros_gz_sim create -world empty -file models/red_block.sdf -name red_block -x 0.25 -y 0.0 -z 0.0205 sleep 20 gz model -m red_block -p # 3. Solve IK offline (moveit_py only, no rclpy) python3 scripts/precompute_ik.py --block-x 0.25 --block-y 0.00 # 4. Bring up MoveIt2's move_group ros2 launch mycobot_moveit_config move_group.launch.py & # 5. Start the recorder and activate it ros2 run rosetta episode_recorder_node --ros-args \ -p contract_path:=contracts/mycobot_pick_place.yaml -p bag_base_dir:=/tmp/my_episode/bag & ros2 lifecycle set --no-daemon --spin-time 15 /episode_recorder configure ros2 lifecycle set --no-daemon --spin-time 15 /episode_recorder activate # 6. Run the sequence python3 scripts/run_pick_and_place.py --record \ --prompt "pick up the red block and place it on the plate" \ --log /tmp/my_episode/grasp_log.csv --meta /tmp/my_episode/grasp_meta.json # 7. Read the verdict cat /tmp/my_episode/grasp_meta.json

For the full 20-episode batch instead: bash scripts/batch.sh 1 20; bash scripts/status.sh reports ground truth.

To convert recorded bags into the dataset: python3 scripts/port_bag.py --out <path> --indices "1 2 3 ... 15".

### Running the launch files instead, by hand

Everything above (steps 1-7) is what Section 14's run_demo.py now does automatically. To run it via the launch files instead:

# Copy the new files into the container if you haven't already docker cp scripts/run_demo.py gazebo_to_lerobot:/workspace/htgpp/run_demo.py docker cp scripts/precompute_ik.py gazebo_to_lerobot:/workspace/htgpp/precompute_ik.py docker cp launch gazebo_to_lerobot:/workspace/htgpp/launch # Headless, exits with a pass/fail code docker exec -it gazebo_to_lerobot bash -c ' source /opt/ros/jazzy/setup.bash; source /workspace/install/setup.bash ros2 launch /workspace/htgpp/launch/pick_and_place_no_gui_demo.launch.py echo "exit code: $?"' # With the GUI (needs a display reachable from the container) docker exec -it gazebo_to_lerobot bash -c ' source /opt/ros/jazzy/setup.bash; source /workspace/install/setup.bash ros2 launch /workspace/htgpp/launch/pick_and_place_demo.launch.py' # Try a different block position ros2 launch /workspace/htgpp/launch/pick_and_place_no_gui_demo.launch.py block_x:=0.28 block_y:=0.05 # See every argument and its description ros2 launch /workspace/htgpp/launch/pick_and_place_no_gui_demo.launch.py --show-args

How to read the result. The launch prints a boxed RESULT line near the end:

[demo 14:19:10] ============================================================ [demo 14:19:10] RESULT: motions_ok=True placed_on_plate=True grasp_held=True [demo 14:19:10] carry mechanism: SIMULATED ATTACHMENT (gz DetachableJoint), NOT a physical grasp [demo 14:19:10] ============================================================

All three of motions_ok, placed_on_plate and grasp_held must be True for a pass. The same fields are also written to demo/grasp_meta.json under the project root.

Checking attach_mode validation. Any value other than simulated must fail immediately:

ros2 launch /workspace/htgpp/launch/pick_and_place_no_gui_demo.launch.py attach_mode:=friction # -> "attach_mode: only 'simulated' is currently supported..." exit code 2, nothing started

Confirming a clean teardown. After the launch exits, no simulation processes should remain:

docker exec gazebo_to_lerobot bash -c 'pgrep -af "gz sim¦move_group¦sim_full"' # nothing printed = clean. <defunct> entries are already-dead zombies -- harmless.

Running this for the first time on a fresh container? Section 10's pre-solved waypoints/ directory isn't needed here: Section 13's live-detection path solves IK for wherever the block actually lands, every time.

Watching the GUI variant on a WSL2 laptop: the plain GUI command above freezes or shows a white window there. Use docker exec -it gazebo_to_lerobot /workspace/htgpp/run_gui_demo.sh instead, after the one-time setup of Sec.17–Sec.23 (see Sec.27).

# File manifest

What's actually tracked in git under this directory, and what each piece is for.

Headless_Task-Grounded_Pick-and-Place_in_Gazebo/ ¦-- doc/ # this report and the source specification ¦ ¦-- headless_pick_and_place_specification.md ¦ `-- PICK_AND_PLACE_REPORT.{html,docx} / _FR.{html,docx} ¦-- MEASUREMENTS.md ¦-- models/ ¦ ¦-- red_block.sdf ¦ `-- plate.sdf ¦-- launch/ # Sec.15, the single ros2 launch entry point ¦ ¦-- pick_and_place_demo.launch.py # gui:=true default, for the manager ¦ `-- pick_and_place_no_gui_demo.launch.py # always headless, pass/fail exit code ¦-- scripts/ ¦ ¦-- grab_frame.py ¦ ¦-- ik_helper.py, precompute_ik.py # Sec.3-4, Sec.13 ¦ ¦-- run_pick_and_place.py ¦ ¦-- episode.sh, batch.sh ¦ ¦-- status.sh ¦ ¦-- port_bag.py ¦ ¦-- run_demo.py # Sec.14, orchestrator behind both launch files ¦ `-- run_gui_demo.sh # Sec.27, one-command GUI demo for WSL2 (2026-10-03) ¦-- contracts/ ¦-- datasets/ `-- .gitignore

The recorded bags themselves (~1.7 GB across 20 episodes) live only in the gazebo_to_lerobot container, not in this repository.

Headless_Task-Grounded_Pick-and-Place_in_Gazebo report | written 2026-09-22 | commit 5c92a565 | Tomislav Ilic | updated 2026-10-03